Self Assessment – Module E: Classification of Mechanical Ventilation

- Input Control can be either:

  - PNEUMATIC

  - ELECTRICAL

  - PNEUMATIC & ELECTRICAL

- Control Variable – Which parameter (waveform) is staying constant regardless of changes in compliance and/or Raw

  - PRESSURE

  - TIME

  - FLOW

  - VOLUME

- Phase Variables correlate to each phase of the ventilatory cycle

  - What starts inspiration TRIGGER

  - What happens during inspiration LIMIT

  - What ends inspiration CYCLE

  - What happens during expiration BASELINE

- How can the ventilator be triggered on?

  - Machine Trigger: TIME (ALSO MANUAL WITH CLINICIAN ACTION)

  - Patient Trigger: PRESSURE OR FLOW

- What parameters can be limited during inspiration?

  - PRESSURE

  - FLOW

  - VOLUME

- How can ventilators be cycled off?

  - TIME

  - PRESSURE

  - FLOW

  - VOLUME

- What parameter is most commonly controlled during expiration? PRESSURE

- What are the baseline variables

  - PEEP

  - NEEP

  - ZEEP

- Name the two breath types: MANDATORY (MACHINE OR MECHANICAL) or SPONTANEOUS.

- Mandatory breaths can be either TIME triggered or PATIENT (FLOW or PRESSURE) triggered.

- A mandatory breath that is patient triggered on is called an ASSISTED breath.

- Name two types of spontaneous breaths.

  - PRESSURE SUPPORTED

  - TUBE COMPENSATION