PERFORMANCE EVALUATION - #28	NAME: _________________________

NON-INVASIVE VENTILATION (BiPAP)			DATE: _________________________

INSTRUCTOR: __________________

ORAL REVIEW QUESTIONS

- Explain the indications for Non-Invasive Ventilation therapy.

  - Alveolar hypoventilation ( PaCO2).

  - Chronic ventilatory muscle dysfunction or fatigue.

  - Must catch early!!!

  - Post extubation difficulty.

  - No code patients who do not wish to be intubated.

  - CHF/Pulmonary Edema.

  - COPD

- Explain contraindications for Non-Invasive Ventilation therapy.

  - Patient incapable of maintaining life sustaining ventilation

  - Apnea

  - Need invasive mechanical ventilation

  - Possibility of aspiration (full face mask).

  - Pneumothorax/pneumomediastinum.

  - Hypotension.

  - Allergy or hypersensitivity to the mask material. (relative)

  - Facial Trauma (relative)

  - Claustrophobia (relative)

  - Untreated pertussis (relative)

  - Epistaxis (relative)

  - Acute sinusitis or otitis media (relative)

- Explain the clinical data and lab data that should be monitored and assessed prior to beginning Non-Invasive Ventilation therapy.

  - ABGs

  - SpO2

  - Level of Consciousness

  - Respiratory rate

  - Blood pressure

- Explain the modes available on the Non-Invasive Ventilation unit.

  - Spontaneous/Timed – Supports each spontaneous breath to augment tidal volume

  - CPAP -  Prevents airway collapse in OSA

- Which controls are active in each mode?

  - CPAP – CPAP and FiO2

  - S/T – IPAP, EPAP, % IT (or inspiratory time), rate, and FiO2

- Differentiate between the IPAP and EPAP controls.

  - IPAP – Sets the level of pressure attained during the inspiratory phase.

  - EPAP – Sets the level of pressure attained during the expiratory phase.

- How is PSV determined?

  - IPAP - EPAP

- Explain the importance of setting the alarms correctly.

  - Allows for improved monitoring of the patient.

- What is considered an “acceptable leak”?

  - 7 to 25 lpm

- Explain when the EPAP level should be increased.

  - To improve oxygenation refractory to changing FiO2

- Explain when the IPAP level should be increased.

  - To improve alveolar ventilation and reduce PaCO2

- Does the IPAP pressure change when you increase EPAP?  What will happen to tidal volume if IPAP is not increased also?

  - No. If EPAP is increased without increasing IPAP proportionately, the PaCO2 will rise.

- Explain how to adjust FiO2 level.

  - According to SpO2 levels.

- How can you assess the adequacy of the supplemental oxygen therapy being provided?

  - SpO2 measurement. (Ignore this question).

- Identify the exhalation port on the device.

  - Unique to interface.

- What flowrate is required to trigger on a breath?

  - Ignore this question.

- What signs would indicate that the mask was putting too much pressure on the patient’s face?

  - Redness of the skin or skin breakdown.

- How would you alleviate the above problem?

  - Adjust straps to minimize pressure

- Does the “Vt” display give a constant number?   Why or why not?

  - No. It varies on a breath-by-breath basis.

- When and how would you wean a patient from NIPPV?

  - When the need for non-invasive ventilation no longer exists. The level of EPAP is decreased as oxygenation improves until you are at a level of 4-5 cm H2O. The level of IPAP is then decreased until you are at or near the level of EPAP.

- What other types of noninvasive devices were used in the pasts? List several.

  - Iron Lung

  - Chest cuirass

  - Pneumobelt

  - Rocking bed

Revised 6/05