# SELF-ASSESSMENT RSPT 2350 - Module C: Sampling Errors

| - Air Bubble with PaO2 less than 160 mm Hg | - Air Bubble with PaO2 greater than 160 mm Hg |
| --- | --- |
| pH ______↑__________ | pH _______↑_________ |
| PaCO2 ____↓__________ | PaCO2 _____↓_________ |
| PaO2 _____↑__________ | PaO2 ______↓_________ |
| - Heparin Dilution | - Venous Admixture |
| pH __SLIGHTLY ↓__________ | pH _______↓_________ |
| PaCO2 _____↔_________ | PaCO2 _____↑_________ |
| PaO2 ______↔______ | PaO2 ______↓_________ |
| - Improper Cooling (Metabolic Effect) |  |
| pH _______↓_________ |  |
| PaCO2 _____↑_________ |  |
| PaO2 ______↓_________ |  |
| - Increased Body Temperature (Hyperthermic or Febrile Patient) |  |
| Temperature Corrected (Actual or in vivo values) | Machine Measurement (37° C) (not corrected) |
| pH ______↓__________ | pH MACHINE MEASURED VALUE |
| PaCO2 ____↑__________ | PaCO2 MACHINE MEASURED VALUE |
| PaO2 _____↑__________ | PaO2 MACHINE MEASURED VALUE |
| - Decreased Body Temperature (Hypothermic patient) |  |
| Temperature Corrected (Actual or in vivo values) | Machine Measurement (37° C) (not corrected) |
| pH ______↑__________ | pH MACHINE MEASURED VALUE |
| PaCO2 ____↓__________ | PaCO2 MACHINE MEASURED VALUE |
| PaO2 _____↓__________ | PaO2 MACHINE MEASURED VALUE |

- GAS LAWS

  - Boyle’s  Law

    - Constant: TEMPERATURE

    - Equation:	P1×V1=P2×V2

    - Relationship: INDIRECT (INVERSE)

    - Clinical Application: BODY PLETHYSMOGRAPHY, SPONTANEOUS VENTILATION

  - Charles’ Law

    - Constant: PRESSURE

    - Equation:	V1T1=V2T

    - Relationship: DIRECT

    - Clinical Application: BTPS/ATPS CONVERSIONS

  - Gay Lussac’s Law

    - Constant: VOLUME

    - Equation:	P1T1=P2T

    - Relationship: DIRECT

    - Clinical Application: EFFECT OF TEMPERATURE ON ABG RESULTS

  - Dalton’s Law - Total pressure of a mixture of gases is equal to the sum of the partial pressures of each constituent gas.

  - Henry’s law - If the temperature of the liquid remains constant, the volume of a gas that dissolves in a liquid equals its solubility coefficient times its partial pressure (that is the gaseous partial pressure above the liquid).