SELF ASSESSMENT – RSPT 2350: MODULE L (Part II)

- Name the four types of volume displacement spirometers:

  - ________________________________________

  - ________________________________________

  - ________________________________________

  - ________________________________________

- Which volume displacement spirometer is referred to as the “gold standard” because it is the most accurate? _______________________

- A rotating drum on which an FVC maneuver is recorded on a graph paper is called ___________________________________________________.

- The biggest problem with volume displacement spirometers is _________.

- With a pressure-differential pneumotach, if the resistance is known and kept constant, then a large pressure change across the resistance will result in

  - An increased flow

  - A decreased flow

  - No change in flow

- The Wright respirometer is an example of which flow sensing spirometer?

  - Heated Wire

  - Anemometer

  - Vortex Shedding

  - Turbines

  - Pitot

- The % error that is acceptable when calibrating a super syringe is

  - 1%

  - 3%

  - 5%

  - 8%

  - 10%

- It would be important to ask the patient to stop smoking for 24 hours before which of the following tests?

  - Methacholine challenge

  - Flow-Volume Loop

  - DLCO

  - Single-Breath N2 Elimination

  - N2 Washout

- During an FVC maneuver, a patient should exhale for  _____ seconds unless a plateau is seen on the volume axis for one second.

  - 2 seconds

  - 4 seconds

  - 6 seconds

  - 8 seconds

- If a patient cannot stand to measure their height, what measurement can be taken to estimate height? __________________________________

- All volumes and flowrates must be measured at:

  - ATPS

  - ATPD

  - BTPS

  - BTPD

  - STPD

- After performing three valid pre-bronchodilator FVC maneuvers, which parameter would you evaluate to determine the need for post-bronchodilator testing?

  - FVC

  - FEV1

  - FEV1/FVC%

  - PEFR

  - FEF25-75%

- A flow measuring spirometer that uses struts to create turbulent flow to interrupt ultrasonic sound waves is

  - Hot-wire Anemometer

  - Pressure-Differential Pneumotach

  - Turbines

  - Pitot

  - Vortex

- Which flow measuring spirometer works on the principle that air flow will cool the surrounding environment?

  - Thermistor

  - Pressure-Differential Pneumotach

  - Turbines

  - Pitot

  - Vortex

- Predicted equations are based on

    - Height

    - Sex

    - Race

    - Age

    - Weight

  - I, II, III

  - II, III, IV

  - I, II, III, IV, V

  - I, II, III, IV

  - I, IV